# Big Data, Mobility Observations & Evaluation

Assigned member: Lee Do Yeong

Purpose: Own the data pipeline, sampling-bias experiments, performance metrics, statistical evaluation and reproducibility evidence.

Scope: Research ownership and implementation handoff. The network dimensions, algorithm settings and schedules are proposed design assumptions, not validated field results.

# Team coordination and responsibilities

| Member | Primary responsibility |
| --- | --- |
| Kim Seong Hyeon | Software Development & System Integration |
| Lee Do Yeong | Big Data, Mobility Observations & Evaluation |
| Lee Yun Seong | AI Algorithms, Robustness & Security |

# Technical ownership

Synthetic demand and scenario design; infrastructure detector feeds; sampled connected-vehicle observations; traffic-state estimation data; ETL, schema, data quality, metrics, statistics and results reporting.

Retain original activity-report responsibilities for effectiveness metrics and data analysis, including Databricks visit questions about geospatial processing costs and MLOps.

# Data model and contracts

Vehicle events: run_id, sim_time_s, vehicle_id_pseudonym, lane_id, position_m, speed_mps, route_id, observed_flag.

Detector intervals: run_id, detector_id, start_s, end_s, count, occupancy, speed, missing_flag, latency_s.

Signal events: junction_id, phase_id, requested_action, executed_action, reason, phase_start_s, phase_end_s.

Scenario metadata: demand_seed, scenario_id, turning ratios, demand levels, incident, weather proxy (if modeled), penetration, sampling mechanism.

Use a documented schema with units, null semantics, timestamp granularity, and retention/privacy policies.

# Observation bias research

Connected-vehicle sampling at the vehicle level for each full trip; test 10%, 25%, 50%, 75%, 100% participation.

Separate uniform random sampling, approach-dependent bias and state/speed-dependent reporting bias.

Do not expose SUMO ground-truth vehicle state to AI controller except in explicitly labeled oracle benchmarks.

Compare estimated queues/flows to ground truth using MAE, RMSE, bias and coverage-stratified errors.

# Traffic metrics and formulas

Primary endpoint: mean vehicle delay (seconds/vehicle) with clearly defined measurement population.

Secondary: p95 corridor travel time, stops/vehicle, queue length, spillback duration, throughput, cross-street delay, arrivals-on-green, unfinished trips and teleports.

Improvement percentage for lower-is-better metric = 100 × (baseline − candidate) / baseline; avoid undefined denominator cases.

Queue occupancy warning: measured queue / nominal storage; actual spillback requires upstream blocking evidence, not just threshold exceedance.

# Experimental design and statistical analysis

Use matched demand seeds across controllers and separate training, tuning and holdout scenarios.

Run preliminary pilot to estimate variance; target at least 20 paired demand seeds for primary comparisons when feasible.

Report paired mean differences, 95% confidence intervals, per-scenario distributions, training-seed variation for RL and multiple-comparison adjustments.

Check for selection bias from unfinished trips and departures blocked by congestion; report flow conservation diagnostics.

Track compute costs, runtime and data volume for Databricks-style scalability discussion; do not claim cloud cost savings without measured billing data.

# Source evidence and verification register

Boston: original report attributes 114 intersections and 20% fewer unnecessary stops / 13.5% lower delay to city announcement and INRIX; treat as reported field findings, not independently proven causal estimates.

Jeju: original report gives 23:21 to 19:31 travel time and 31.3% lower delay; primary technical evaluation and sampling details remain to be obtained.

Busan: original report gives delay 13.9 to 12.5 seconds; obtain underlying municipal study before treating as validated causal result.

Record source owner, publication date, baseline, sample, metric definition, comparison period, confounders, independence and missing details.

# Definition of done

Documented datasets and data dictionary; sampling generator; scenario matrix; reproducible KPI extraction; paired analysis notebook; evidence matrix and publication-quality result tables.

# Cross-team integration checkpoints

Checkpoint 1: agree on SUMO geometry, phase-state indexing, detector schema and shared scenario IDs.

Checkpoint 2: run identical baseline scenario with common KPI definitions and confirmed safety checks.

Checkpoint 3: freeze holdout scenarios before final algorithm selection and report paired outcomes.

Checkpoint 4: jointly review evidence limitations, operational risks and presentation claims.

# Reference material and provenance

The uploaded Team B Activity Report dated 13 October 2026 is the source for original member roles, research questions and reported case-study figures. The expanded technical assignments, architecture and tests in this document are proposed by the research team and remain to be implemented.

Team B Activity Report (13 Oct 2026): User-provided REPORT_D.pdf; primary source for original team roles, case claims and questions.

SUMO documentation: https://sumo.dlr.de/docs/

FHWA Traffic Signal Timing Manual: https://ops.fhwa.dot.gov/publications/fhwahop08024/

NCHRP Signal Timing Manual, 2nd ed.: https://nap.nationalacademies.org/catalog/22097/signal-timing-manual-second-edition

Boston Project Green Light announcement: https://www.boston.gov/news/mayor-wu-announces-expansion-project-green-light-signal-optimization-program

Google Project Green Light: https://sites.research.google/greenlight/

PressLight (KDD 2019): https://doi.org/10.1145/3292500.3330949

CoLight (CIKM 2019): https://doi.org/10.1145/3357384.3357902

RESCO benchmark: https://github.com/Pi-Star-Lab/RESCO

SUMO-RL: https://github.com/LucasAlegre/sumo-rl