# Software Development & System Integration

Assigned member: Kim Seong Hyeon

Purpose: Build the executable, testable four-intersection SUMO platform and integrate all modules into a reproducible demonstration.

Scope: Research ownership and implementation handoff. The network dimensions, algorithm settings and schedules are proposed design assumptions, not validated field results.

# Team coordination and responsibilities

| Member | Primary responsibility |
| --- | --- |
| Kim Seong Hyeon | Software Development & System Integration |
| Lee Do Yeong | Big Data, Mobility Observations & Evaluation |
| Lee Yun Seong | AI Algorithms, Robustness & Security |

# Technical ownership

SUMO network and traffic signal programs; Python/TraCI simulation orchestration; shared controller API; independent safety supervisor; CI tests; experiment runner; dashboard and packaging.

Retain original activity-report responsibility for overall coordination, overseas case research, Google visit questions, and final presentation.

# Reference system design

Four junctions J1–J4 at x=0,300,600,900 m; two-way east–west corridor, cross-street approaches, explicit connections and valid turn permissions.

1 s simulation step; 900 s warm-up; 3600 s measurement; 90 s baseline cycle as provisional assumptions, not field-calibrated values.

Use SUMO netconvert and validate traffic-light controlled-link indices from generated network, never hard-code signal state order.

# Software architecture

network/ generated XML and net files; demand/ route generator; controllers/ fixed, actuated, coordinated, max-pressure, MPC, MARL; safety/; sensors/; evaluation/; tests/; results/.

Controller interface: reset(config, seed), observe(observation), propose_action(state), supervisor.validate(action), apply_safe_action(action), log(step).

Produce JSON experiment manifests including code revision, SUMO version, demand hash, network hash, seeds, controller parameters and output locations.

# Implementation sequence

1. Generate four-junction network, demand scenarios, legal routes, and fixed-time signal programs.

2. Validate controlled connections, phase conflicts, cycle transitions and route completion.

3. Integrate detector observations and data-team interfaces; implement baseline fixed-time and actuated controllers.

4. Add optimized coordinated controller and max-pressure; integrate AI team controllers behind the same safety API.

5. Run paired scenarios and publish reproducible CSV/Parquet results plus SUMO-GUI demo.

# Safety and acceptance tests

NET-01: four junctions, valid two-way and cross-street routes, no unexpected teleport errors.

SIG-01: zero conflicting protected greens; enforce minimum greens, required yellow/all-red and pedestrian service when modeled.

SIG-02: reject invalid proposed actions and log reason; safe fallback when controller fails or times out.

REP-01: repeated runs with identical configuration and seeds reproduce outputs within documented deterministic tolerance.

PERF-01: report completed and unfinished trips so gridlock cannot bias apparent success.

# Handoffs and dependencies

From Big Data: detector schema, observation masks, demand scenario manifests, metric definitions and validation datasets.

From AI: controller adapter, model dependencies, action specification, inference-time budget and trained artifacts.

To both: stable SUMO environment, safety supervisor, experiment harness and baseline performance outputs.

# Definition of done

One-command reproducible baseline experiment on all four junctions; fixed-time and coordinated controller comparison; validated signal transitions; exported results; setup README; demo recording or GUI walkthrough.

# Cross-team integration checkpoints

Checkpoint 1: agree on SUMO geometry, phase-state indexing, detector schema and shared scenario IDs.

Checkpoint 2: run identical baseline scenario with common KPI definitions and confirmed safety checks.

Checkpoint 3: freeze holdout scenarios before final algorithm selection and report paired outcomes.

Checkpoint 4: jointly review evidence limitations, operational risks and presentation claims.

# Reference material and provenance

The uploaded Team B Activity Report dated 13 October 2026 is the source for original member roles, research questions and reported case-study figures. The expanded technical assignments, architecture and tests in this document are proposed by the research team and remain to be implemented.

Team B Activity Report (13 Oct 2026): User-provided REPORT_D.pdf; primary source for original team roles, case claims and questions.

SUMO documentation: https://sumo.dlr.de/docs/

FHWA Traffic Signal Timing Manual: https://ops.fhwa.dot.gov/publications/fhwahop08024/

NCHRP Signal Timing Manual, 2nd ed.: https://nap.nationalacademies.org/catalog/22097/signal-timing-manual-second-edition

Boston Project Green Light announcement: https://www.boston.gov/news/mayor-wu-announces-expansion-project-green-light-signal-optimization-program

Google Project Green Light: https://sites.research.google/greenlight/

PressLight (KDD 2019): https://doi.org/10.1145/3292500.3330949

CoLight (CIKM 2019): https://doi.org/10.1145/3357384.3357902

RESCO benchmark: https://github.com/Pi-Star-Lab/RESCO

SUMO-RL: https://github.com/LucasAlegre/sumo-rl